Final year project:

Final Year Project — Research Proposal

Proposed Title: Investigating Ensemble Uncertainty for Failure Detection and Intervention in Robotic Manipulation

Research Aim

This project investigates whether ensemble-based epistemic uncertainty can identify potential failures in robotic manipulation, particularly when a robot encounters cluttered environments outside its training distribution.

Building upon the ensemble-disagreement approach in Refine-Plan, the project will investigate three central questions:

- Does epistemic uncertainty increase because an environment is unfamiliar, even when manipulation failure risk remains similar?

- Does epistemic uncertainty indicate when the ensemble's predictions are becoming inaccurate, and can it help identify actions that will fail?

- Can using epistemic uncertainty alongside predicted failure probability improve robotic manipulation through selective intervention?

Stage 1 — Simulation Environment and Manipulation

Objective: Build a simulated robot capable of executing reliable manipulation actions.

Implementation:

- Robot: Franka Emika Panda robotic arm.

- Simulator: MuJoCo for physics and robosuite for robotic manipulation.

- Interface: Python and Gymnasium.

- Environment: Tabletop containing target objects, distractors, and an overhang that can obstruct certain grasps.

- Actions: Scripted top, left and right grasps, pick-and-place operations, and relocation of obstructing objects.

Observations:

- The robot receives numerical information about object positions, dimensions, and task status.

- Gaussian noise is added to object-position estimates to simulate imperfect perception.

- Computer vision is not required initially.

Initial validation:

- Verify that unobstructed grasps reliably succeed.

- Confirm that meaningful physical failures occur, including collisions, misses, and drops.

- Test reproducibility using fixed random seeds and measure simulation speed.

Deliverable: A functioning robotic manipulation environment capable of generating successful and unsuccessful actions.

Stage 2 — Dataset Collection and Outcome Ensemble

Objective: Train models to predict the probability that a manipulation action will succeed.

Dataset collection:

- Execute actions under different object arrangements and clutter configurations.

- Initially select actions using:

  - Random policy: Randomly chooses from valid actions.

  - Geometric heuristic: Selects actions using hand-designed rules based on object positions and grasp clearance.

- Record each observation, action, and outcome:

[
(o_{t},a_{t},y_{t}),\qquad y_{t}\in{0,1}
]

where (y_{t}=1) indicates success and (y_{t}=0) indicates failure.

Model training:

- Train five independently initialised neural networks using PyTorch.

- Each network predicts the probability of success given observation (o) and action (a):

[
p_{i}=P_{i}(Y=1\mid o,a)
]

Uncertainty measures:

- Mean predicted success probability:

[
\bar{p}=\frac{1}{K}\sum_{i=1}^{K}p_{i}
]

- Predicted failure probability:

[
\hat{P}_{\text{fail}}=1-\bar{p}
]

- Epistemic uncertainty (ensemble disagreement):

[
U_{\text{epi}}=H(\bar{p})-\frac{1}{K}\sum_{i=1}^{K}H(p_{i})
]

where (H) is binary entropy and (K=5).

Deliverable: Five trained models that provide estimated action-success probabilities and measures of uncertainty.

Stage 3 — Reinforcement Learning Policy

Objective: Train a policy to make useful sequential manipulation decisions.

Implementation:

- Train a MaskablePPO reinforcement learning agent.

- The agent selects which object to manipulate and which grasp approach to use.

- Actions are executed using the scripted manipulation primitives from Stage 1.

Curriculum learning:

- Begin with simple, unobstructed pick-and-place tasks.

- Introduce obstacles and alternative grasp approaches.

- Progress to environments where obstructing objects must be relocated before target objects can be grasped.

Evaluation:

- Compare PPO against geometric-heuristic and greedy policies.

- Measure task success rate, collisions, decisions per episode, and performance on tasks requiring obstacle relocation.

Deliverable: A trained policy capable of proposing manipulation actions, including multi-step decisions.

Stage 4 — Uncertainty Analysis, Failure Detection and Generalisation

Objective: Determine whether epistemic disagreement measures environmental novelty, model prediction error, or actual manipulation failure risk.

Experiment 4.1 — Does unfamiliarity increase epistemic uncertainty?

Procedure:

- Train the ensemble using scenes containing 0–4 distractors (familiar environments).

- Evaluate on unseen scenes containing 6–8 distractors (unfamiliar/OOD environments).

- Allow the trained PPO policy to propose actions in both environments.

- For selected actions, repeat the same proposed action approximately 100 times from the same scene geometry, while resampling specified physical and execution noise.

- Estimate the action's underlying success probability:

[
\hat{q}(s,a)=\frac{\text{Successful trials}}{\text{Total trials}}
]

- Compare familiar and unfamiliar actions with approximately equal estimated failure probabilities, (1-\hat q(s,a)), while accounting for sampling uncertainty.

Example:

- Familiar action: estimated success probability = 80%, epistemic uncertainty = 0.02.

- Unfamiliar action: estimated success probability = 80%, epistemic uncertainty = 0.15.

Although both actions have approximately the same physical failure risk, the ensemble disagrees more in the unfamiliar environment.

Research question: Does epistemic disagreement increase simply because the environment is unfamiliar, even when the underlying failure risk is comparable?

Metric: Difference in epistemic uncertainty between familiar and unfamiliar actions matched on estimated failure risk.

Experiment 4.2 — Does epistemic uncertainty indicate prediction error?

Procedure:

- For each action from Experiment 4.1, record:

  - The ensemble's mean predicted success probability, (\bar{p}).

  - Its epistemic uncertainty, (U_{\text{epi}}).

  - Its estimated geometry-conditional success probability, (\hat q(s,a)).

- Calculate the squared discrepancy:

[
E=(\bar{p}-\hat{q}(s,a))^2
]

- Apply an appropriate correction for uncertainty caused by the finite number of repeated trials.

- Compare the discrepancies between familiar and unfamiliar environments, and investigate whether larger (U_{\text{epi}}) is associated with larger discrepancies.

Possible outcomes:

- Higher disagreement and larger errors: Epistemic uncertainty may provide a useful warning that model predictions are becoming unreliable.

- Higher disagreement but similar errors: The ensemble may be responding to unfamiliarity without becoming less accurate.

- Low disagreement but large errors: The ensemble may be confidently wrong, exposing a limitation of ensemble-based uncertainty.

Research question: Does epistemic disagreement indicate when the ensemble's predictions are inaccurate, or does it primarily reflect novelty?

Metric: Squared prediction discrepancy and its relationship with (U_{\text{epi}}), accounting for repeated-rollout uncertainty.

Important distinction: The ensemble predicts from noisy observations, whereas (\hat q(s,a)) conditions on true scene geometry. Therefore, the discrepancy is not necessarily entirely model error; the analysis will account for uncertainty caused by information unavailable to the models.

Experiment 4.3 — Can our metrics identify actions that will fail?

Procedure:

- Run the trained PPO policy in familiar and unfamiliar test environments without interventions.

- For each proposed action, calculate:

  - Predicted failure probability, (\hat{P}_{\text{fail}}).

  - Epistemic uncertainty, (U_{\text{epi}}).

  - Total predictive uncertainty, (U_{\text{total}}=H(\bar{p})).

- Execute the action and record whether it succeeds or fails.

- Compare the signals using the following metrics:

| Metric | Purpose |
| --- | --- |
| AUROC | Measures how well a score ranks actions that fail above actions that succeed, across thresholds |
| AUPRC | Evaluates failure detection through precision and recall, particularly useful when failures are relatively rare |
| Calibration | Determines whether predicted probabilities agree with observed failure frequencies |
| Brier score | Measures the accuracy of predicted failure probabilities against observed outcomes |

Research question: Which uncertainty measure is most effective at identifying manipulation actions that will fail, particularly under distribution shift?

Deliverable for Stage 4: A controlled evaluation of whether ensemble disagreement indicates novelty, prediction inaccuracy, or manipulation failure risk.

Stage 5 — Uncertainty-Gated Intervention

Objective: Determine whether uncertainty information can improve manipulation success by selectively overriding the PPO policy.

Intervention mechanism:

- PPO proposes a manipulation action.

- The ensemble evaluates the proposed action.

- An intervention score is calculated.

- If the score exceeds a specified threshold, the PPO action is replaced with an alternative selected by a fixed geometric heuristic.

- Otherwise, the proposed PPO action is executed.

The same geometric fallback will be used for every intervention strategy.

Intervention strategies

| Strategy | Intervention trigger |
| --- | --- |
| No intervention | PPO always executes its proposed action |
| Random intervention | Randomly selected decisions |
| Predicted failure probability | High (\hat{P}_{\text{fail}}) |
| Epistemic uncertainty | High (U_{\text{epi}}) |
| Total uncertainty | High (U_{\text{total}}) |
| Combined risk–uncertainty | High combined risk score |

The combined score is defined as:

[
R_{\lambda}(o,a)=\hat{P}{\text{fail}}(o,a)+\lambda\sigma{\text{ensemble}}(o,a)
]

where (\sigma_{\text{ensemble}}) represents the standard deviation of the five success-probability predictions and (\lambda) controls the weight assigned to disagreement.

Experimental design:

- Compare intervention budgets targeting 5%, 10%, and 20% of decisions.

- Calibrate thresholds so strategies intervene at approximately comparable rates.

- Evaluate performance on the same initial test scenes.

- Additionally, test thresholds calibrated on familiar scenes and applied unchanged to unfamiliar scenes.

Evaluation metrics:

| Metric | Purpose |
| --- | --- |
| Episode success rate | Does intervention improve overall task completion? |
| Intervention frequency | How often is PPO overridden? |
| Manipulation failure rate | Do interventions prevent unsuccessful actions? |
| Collision frequency | Do interventions reduce unwanted physical interactions? |
| Success at matched budgets | Which trigger uses a limited number of interventions most effectively? |

Research question: Does incorporating epistemic uncertainty into intervention decisions improve manipulation success beyond predicted failure probability alone?

Deliverable: A comparison of intervention strategies, identifying whether epistemic uncertainty provides practical value for robotic manipulation.

Expected Contribution and Feasibility

The primary contribution is a controlled empirical investigation into what ensemble disagreement measures when robotic manipulation systems encounter unfamiliar environments.

The project will establish whether disagreement reflects environmental novelty, unreliable predictions, or actual failure risk, and whether it provides additional information beyond predicted failure probability.

A secondary contribution is determining whether this information can improve task success through selective intervention.

Experiments will use separate training and testing environments, repeated random seeds, and confidence intervals. A simplified version using a geometric-heuristic acting policy will allow the central uncertainty experiments to proceed independently of PPO convergence.